🏋️ SmartFit Trainer

IoT-Based Sports Performance & Health Monitor

ESP32  +  Raspberry Pi  +  MQTT  +  Grafana

# Project Overview

SmartFit Trainer is a real-time fitness monitoring system that tracks an athlete's vital signs, movement quality, and environmental conditions during exercise. The system uses an ESP32 microcontroller connected to multiple sensors, communicates over MQTT via a Raspberry Pi broker, and visualizes all data on a live Grafana dashboard.

The project demonstrates a complete Wireless Sensor Network (WSN) pipeline — from physical sensor data acquisition, through wireless transmission, to cloud-style visualization and alerting.

# Project Goals

- Monitor heart rate (BPM) and detect training heart rate zones in real-time

- Track body temperature and environmental heat stress during exercise

- Count reps and detect impact force on joints using shock/vibration sensors

- Alert the user when BPM exceeds safe thresholds via buzzer and RGB LED

- Log all session data over MQTT for post-workout analysis in Grafana

- Control a cooling fan automatically when body temperature rises

# Sensors & Components Used

| Sensor | Purpose | Output |
| --- | --- | --- |
| Heartbeat Sensor | BPM + HRV monitoring | Analog → ADC |
| Analog Temp | Body temperature | Analog → ADC |
| Temp + Humidity (DHT) | Environment heat stress | Digital |
| Shock Sensor × 2 | Step count + impact force | Digital interrupt |
| Tilt Switch | Exercise angle / form | Digital |
| LightBlocking Sensor | Rep counter (beam break) | Digital |
| Active Buzzer | Heart rate zone alert | Digital output |
| Passive Buzzer | Interval timer / beeps | PWM output |
| RGB LED | HR zone color indicator | PWM (R/G/B pins) |
| Relay | Auto fan control | Digital output |
| Rotary Encoder | Set target HR / rest time | Digital (A/B/SW) |
| Joystick | Menu navigation | Analog X/Y + button |
| Touch Tracking | Start / stop session | Digital |
| Microphone (Small) | Detect yawning / fatigue | Analog |

# System Architecture

The system follows a 3-layer WSN architecture:

- Perception Layer: ESP32 collects data from all 14 sensors at 10Hz

- Network Layer: ESP32 publishes data over Wi-Fi using MQTT protocol

- Application Layer: Raspberry Pi runs Mosquitto broker, Node-RED, InfluxDB, and Grafana

MQTT Topic Structure:

- smartfit/vitals/bpm  →  Heart rate

- smartfit/vitals/temp  →  Body temperature

- smartfit/environment  →  Humidity + ambient temp

- smartfit/motion/steps  →  Step count

- smartfit/motion/impact  →  Shock force

- smartfit/alerts/zone  →  Current HR zone (1–5)

# Heart Rate Zone Logic

| Zone | BPM Range | LED Color | Action |
| --- | --- | --- | --- |
| Zone 1 — Rest | < 100 BPM | Blue | Fan OFF |
| Zone 2 — Fat Burn | 100–120 BPM | Green | Fan ON low |
| Zone 3 — Cardio | 120–140 BPM | Yellow | Fan ON high |
| Zone 4 — Hard | 140–160 BPM | Orange | Buzzer warning |
| Zone 5 — Max | > 160 BPM | Red | Buzzer alarm + alert |

# Technology Stack

| Sensor | Purpose | Output |
| --- | --- | --- |
| Firmware | ESP32 Arduino / ESP-IDF | FreeRTOS tasks |
| Protocol | MQTT (paho) | Wi-Fi 2.4GHz |
| Broker | Mosquitto on RPi | Raspberry Pi 4 |
| Pipeline | Node-RED | Data routing + alerts |
| Database | InfluxDB | Time-series storage |
| Dashboard | Grafana | Live visualization |
| Alerts | Telegram Bot API | Push notifications |

# Development Timeline

- Week 1: Hardware setup, sensor wiring, basic ESP32 firmware

- Week 2: MQTT integration, Raspberry Pi broker setup, Node-RED pipeline

- Week 3: Heart rate zone logic, buzzer/LED/relay feedback system

- Week 4: Grafana dashboard, Telegram alerts, full system testing

- Week 5: Calibration, performance tuning, documentation & demo

SmartFit Trainer — IoT Project Proposal  |  ESP32 + Raspberry Pi + MQTT